PERFORMANCE EVALUATION OF TACTOR POWER TAKE OFF (PTO) CUTTING MECHINSM FOR SUGARCANE PLANTER

Dr. Ramesh Pal* Er. Amar Sigh & Mr. Abhishek Dubey

School of Agricultural Sciences and Engineering

IFTM, University, Moradabad, Uttar Pradesh, India, Pin: -244102

SCRIET, CCS University Campus, Meerut

*Corresponding Author Email: -ramesh.ift@gmail.com

# Abstract

Sugarcane planting is a very labour-intensive job and involves considerable human drudgery. Cost of sugarcane planting by mechanized method is less compared to traditional methods. It also reduces drudgery involved in unit operations of sugarcane planting. About 13.25 percent of the World’s and 41.11 percent of Asian sugar production and it is cultivated in 4.10 million hectares with annual production of 300.25 million tonnes of cane and 18.90 million of sugar by India

The present study describes the different parameters like the length of sets, no of sets, no of damaged buds, the gap between two sets, field efficiency, etc by the PTO operated sugarcane cutting mechanism planter   at three rpm (1250,1350 and 1450) . Results show that the field. Its field capacity was 0.2-0.22 ha/hr with field efficiency of 85.0- 85.0 % at an effective working width of 0.90 m and forward speed of 2.0-2.20 km/hr. The average set length was 33.0 cm with an average overlap of 6.15-5.30 cm observed and seed rate 5.6 - 7.1 t/ha.

Keywords: Sugarcane, Semiautomatic Planter, Wholesale Planter, Set Cutting Blade tractor operated, India